ASM48 CROSS-ASSEMBLER V1.2          25 Sep 02

ASM48  is a CP/M cross-assembler for the INTEL MCS-48 and  UPI-41

series of 8 bit single-chip micro-controllers.  These include the

8035,  8039,  8048,  8748 and 8041 as well as others.  Written in

8080 machine language,  the assembler will run on all CP/M-80 and

compatible operating systems.

1. DESCRIPTION

ASM48  is based on the well-known Digital Research assembler  ASM

and its derivative,  LASM by Ward Christensen.   These assemblers

were intended to assemble 8080 programs and have been widely used

throughout the CP/M community.

Though ASM48 assembles source code for a different processor, the

command-line options,  psuedo-opcodes and error messages are  the

same as that for ASM.   The user's task of learning a new set  of

assembler syntax conventions is thus considerably minimised.

In addition to all the usual ASM psuedo-opcodes,  ASM48  includes

the TITLE,  PAGE and ELSE directives.   IF ELSE ENDIF  statements

may be nested (up to 8 levels).

For CP/M 3.x or ZCPR 3 users,  ASM48 sets the Program Return Code

indicating  whether  or  not  any  errors  were  detected  during

assembly.   This may be used to control program flow within batch

scripts.

2. INSTALLATION

ASM48  does not require installation and may be used immediately.

There  are several parameters which the user may wish to alter to

suit  his  or  her  particular  needs.   These  control  how  the

assembler operates.   Changes are made by altering specific bytes

within a patch area located near the beginning of ASM48.COM using

DDT,  SID  or similar utility.   Alternatively,  the source  file

for ASM48 may be altered and then re-assembled.

Address

010B  Initial Form-Feed

This  byte  controls whether a form-feed character is to  be

sent at the start of a listing file.  A zero at this address

indicates that a form-feed is NOT sent.  Default = 00h

010C  Ending Form-Feed

This  byte  controls whether a form-feed character is to  be

sent at the end of a listing file.   A zero at this  address

indicates that a form-feed is NOT sent.  Default = 0FFh

010D  Lines per Page

This  byte indicates the number of output lines per page for

the listing file.   The default value is 60 (which assumes a

standard  page  length  of 11 inches with text spaced  at  6

lines per inch).   If a value of zero is specified,  no page

breaks or page headers will be output.

Note  - the PAGE <exp> directive may be used to overide this

parameter.

010E  ASM-Style Condition Test

DRI's  ASM tests conditional statements for TRUE or FALSE by

checking only the least significant bit of the  expression's

16 bit value.  If ASM compatibility is desired set this byte

to a non-zero value.  Default = 00h.

010F  ASM-Style Command Line

A  non-zero value at this location causes the  assembler  to

accept  command line options strictly in accordance with ASM

conventions.  A blank filetype on the command line indicates

that  the  source file is located on the default  drive  and

that HEX and PRN files will be generated.  Default = 00h.

3. USAGE

ASM48  assembles  an 8048/41 assembly language source  file  from

disk and produces output in the form of an INTEL HEX ASCII  file.

A listing file may also be generated.

To distinguish them from CP/M ASM files,  8048/41 assembly source

files must have a filetype of 'A48'.

Using  the  assembler is identical to using ASM.   Refer  to  the

assembler reference section.

Note  - the assembler will recognise and assemble mnemonics  from

either the MCS-48 or UPI-41 families.   While each has a  similar

instruction set,  there are differences.  The user must take care

to  use  only the correct mnemonic instruction for the device  in

question.

4. EPROM PROGRAMMING

After the 8048 source file has been assembled without error,  the

user is left with an INTEL HEX file.   At this point, he/she will

want  to  transfer  this  to  an eprom  for  testing  in  a  8048

microprocessor system.

Most  commercial  eprom programmers will be able  to  handle  the

INTEL  HEX  format directly.   In this case,  simply  follow  the

instructions given in the eprom programmer manual.

Should  the  user have an eprom programmer which accepts  only  a

binary  file input,  it will be necessary to translate the  INTEL

HEX  file into this format.   This will require a loader program.

Unfortunately the CP/M program LOAD.COM cannot be used as 8048/41

roms  usually begin with an origin address of 0000H but  LOAD.COM

only allows loading at addresses of 100H or above.

Two solutions present themselves.

a.  Use  Ron Fowler's MLOAD program.   This will create a  binary

object file.   Note that MLOAD automatically gives the binary

file a filetype of COM.   This user should rename this to OBJ

(or other filetype) to prevent execution under CP/M.

b.  Use one of the CP/M debuggers,  DDT,  SID, ZSID etc.  In this

case  follow the method below.   Warning - an offset of  100H

must be specified when reading in the HEX file.

A>ddt

DDT VERS 2.2

-itest.hex          name of hex file to load

-r100               read file with 100h offset

NEXT  PC

024B 0000           size of object (014Bh bytes)

-g0                 exit

The  INTEL HEX file has been converted to a binary image  and

is located at 0100H in memory.   This should now be saved  to

disk using CP/M's SAVE command.

A>save 2 test.obj   save binary image (2 pages)

5. ASM48 ASSEMBLER REFERENCE

5.1  Command Line Format

ASM48 filename.abc

where

a    Drive (A-P) containing the source file.  The assembler

assumes there is a source file on disk with the name:

filename.A48

b    Destination drive (A-P) for the .HEX file.  Z skips the

generation of the .HEX file.

c    Destination drive (A-P) for the .PRN file.  X sends the

.PRN file to the console.  Z skips the generation of the

.PRN file.

Note - unlike ASM, if no filetype is specified the source file is

presumed  to be located on the default drive and no output  files

are generated.   This is especially convenient during testing  as

the  only output is a listing to the screen of those lines  which

contain errors.  (This is optional - refer to section 2.)

5.2  Labels

A  label may consist of any sequence of (up to 16)  alpha-numeric

characters.   All  characters  are  significant  except  for  the

embedded  $ character,  which can be used to improve  legibility.

Labels  must  begin  with an alphabetic  character.   ASM48  also

allows labels to begin with the graphic characters ? or @.

5.3  Numeric Constants

B       Binary constant (base 2)

D       Decimal constant (base 10)

H       Hexadecimal constant (base 16)

O       Octal constant (base 8)

Q       Octal constant (base 8)

5.4  Reserved Words

@A      @R0     @R1     A       BUS     C       CLK     CNT

DBB     DMA     F0      F1      FLAGS   I       MB0     MB1

P1      P2      P4      P5      P6      P7      PSW     R0

R1      R2      R3      R4      R5      R6      R7      RB0

RB1     STS     T       TCNT    TCNTI

5.5  Arithmetic and Logical Operators

+       -       *       /       MOD     NOT     AND     OR

XOR     SHL     SHR

5.6  Assembler Directives

ORG       Set the program or data origin.

END       End program.

EQU       Numeric equate.

SET       Numeric set.

IF        Begin conditional assembly.

ELSE      Conditional assembly.

ENDIF     End conditional assembly.

DB        Define data bytes.

DW        Define data words.

Note - the order of bytes stored is reversed from  ASM.

The most significant byte is stored first,  followed by

the least significant byte.

DS        Define data storage area.

TITLE str Inserts  the ascii string delimited by apostrophes into

the title line of each page of the listing file.

PAGE exp  If given without an expression,  a form-feed is sent to

the  listing  file after the PAGE  statement  has  been

printed.    If   an  expression  is   specified,   this

determines the number of lines per page.   The  default

value  is  60  lines  per  page.    If  the  expression

evaluates  to zero,  no page breaks are issued (this is

useful  when it is desired to generate a  listing  file

without  headers  or breaks for manipulation by a  word

processor etc.).

5.7  Operation Codes

ADD     ADDC    ANL     ANLD    CALL    CLR     CPL     DA

DEC     DIS     DJNZ    EN      ENT0    IDL     IN      INC

INS     JB      JC      JF0     JF1     JMP     JMPP    JNC

JNI     JNIBF   JNT0    JNT1    JNZ     JOBF    JT0     JT1

JTF     JZ      MOV     MOVD    MOVP    MOVP3   MOVX    NOP

ORL     ORLD    OUT     OUTL    RET     RETR    RL      RLC

RR      RRC     SEL     STOP    STRT    SWAP    XCH     XCHD

XRL

5.8  Error Codes

B  Balance error: IF ELSE ENDIF statements not paired.

C  Comma error: a comma was expected.

D  Data error: element in data statement is incompatible.

E  Expression error: expression is ill-formed.

L  Label error: label not allowed or duplicate label.

N  Not implemented: function not implemented.

O  Overflow error: expression is too complex.

P  Phase error: label does not have the same value on each pass.

R  Register error: register is incompatible with the operation.

S  Syntax error: statement not properly formed.

U  Undefined error: operand has not been defined elsewhere.

V  Value error: operand is improperly formed.

6.  THE INTEL MCS-48 AND UPI-41 FAMILY.

The  MCS-48 and UPI-41 microcomputers are  designed  specifically

for controller applications.   They combine, in a single package,

an 8-bit CPU,  built-in oscillator and clock, ROM  (masked, eprom

or  external),  RAM,  27 input/output lines and a  counter/timer.

The  instruction  set consists of over 90  instructions,  70%  of

which are single byte.   A number of devices are included in  the

family and are listed below.

MCS-48 Micro-Controllers

8035       64 x 8 ram   4k external rom

8039      128 x 8 ram   4k external rom

8040      256 x 8 ram   4k external rom

8048       64 x 8 ram   1k x 8 masked rom

8049      128 x 8 ram   2k x 8 masked rom

8050      256 x 8 ram   4k x 8 masked rom

8748       64 x 8 ram   1k x 8 eprom

8749      128 x 8 ram   2k x 8 eprom

UPI-41 Universal Peripherals

8041       64 x 8 ram   1k x 8 masked rom

8741       64 x 8 ram   1k x 8 eprom

7. SUMMARY OF THE MCS-48 AND UPI-41 INSTRUCTION SET

(reference: INTEL Microcontroller Handbook 1986)

A      accumulator

addr   12-bit program memory address

Bb     bit designator (0-7)

BUS    bus port

C      carry

CLK    clock

CNT    event counter

data   8-bit number or expression

DBB    data bus buffer

F0     flag 0

F1     flag 1

I      interrupt

Pp     port designator (p=1,2 or 4-7)

PSW    program status word

Ri     data memory pointer (i=0 or 1)

Rr     register designator (r=0-7)

STS    status register

T      timer

T0     test 0

T1     test 1

*      instructions applicable only to the MCS-48 family

**     instructions applicable only to the UPI-41 family

Accumulator

ADD    A,Rr           Add register to A

ADD    A,@Ri          Add data memory to A

ADD    A,#data        Add immediate to A

ADDC   A,Rr           Add register with carry

ADDC   A,@Ri          Add data memory with carry

ADDC   A,#data        Add immediate with carry

ANL    A,Rr           And register to A

ANL    A,@Ri          And data memory to A

ANL    A,#data        And immediate to A

ORL    A,Rr           Or register to A

ORL    A,@Ri          Or data memory to A

ORL    A,#data        Or immediate to A

XRL    A,Rr           Exclusive Or register to A

XRL    A,@Ri          Exclusive Or data memory to A

XRL    A,#data        Exclusive Or immediate to A

INC    A              Increment A

DEC    A              Decrement A

CLR    A              Clear A

CPL    A              Complement A

DA     A              Decimal adjust A

SWAP   A              Swap nibbles of A

RL     A              Rotate A left

RLC    A              Rotate A left through carry

RR     A              Rotate A right

RRC    A              Rotate A right through carry

Input/Output

IN     A,Pp           Input port to A

OUTL   Pp,A           Output A to port

ANL    Pp,#data       And immediate to port

ORL    Pp,#data       OR immediate to port

IN     A,DBB          Input DBB to A, clear IBF        **

INS    A,BUS          Input BUS to A                   *

OUT    DBB,A          Output A to DBB, set OBF         **

OUTL   BUS,A          Output A to BUS                  *

ANL    BUS,#data      And immediate to BUS             *

ORL    BUS,#data      Or immediate to BUS              *

MOV    STS,A          A4-7 to bits 4-7 of status       **

MOVD   A,Pp           Input expander port to A

MOVD   Pp,A           Output A to expander port

ANLD   Pp,A           And A to expander port

ORLD   Pp,A           Or A to expander port

Registers

INC    Rr             Increment register

INC    @Ri            Increment data memory

DEC    Rr             Decrement register

Branch

JMP    addr           Jump unconditional

JMPP   @A             Jump indirect

DJNZ   Rr,addr        Decrement register and jump

JC     addr           Jump on carry = 1

JNC    addr           Jump on carry = 0

JZ     addr           Jump on A zero

JNZ    addr           Jump on A not zero

JT0    addr           Jump on T0 = 1

JNT0   addr           Jump on T0 = 0

JT1    addr           Jump on T1 = 1

JNT1   addr           Jump on T1 = 0

JF0    addr           Jump on F0 = 1

JF1    addr           Jump on F1 = 1

JTF    addr           Jump on timer flag = 1

JNI    addr           Jump on INT = 0                  *

JNIBF  addr           Jump on IBF = 0                  **

JOBF   addr           Jump on OBF = 1                  **

JBb    addr           Jump on Accumulator bit

Subroutine

CALL   addr           Jump to subroutine

RET                   Return

RETR                  Return and restore status

Flags

CLR    C              Clear carry

CPL    C              Complement carry

CLR    F0             Clear flag 0

CPL    F0             Complement flag 0

CLR    F1             Clear flag 1

CPL    F1             Complement flag 1

Data Moves

MOV    A,Rr           Move register to A

MOV    A,@Ri          Move data memory to A

MOV    A,#data        Move immediate to A

MOV    Rr,A           Move A to register

MOV    @Ri,A          Move A to data memory

MOV    Rr,#data       Move immediate to register

MOV    @Ri,#data      Move immediate to data memory

MOV    A,PSW          Move PSW to A

MOV    PSW,A          Move A to PSW

XCH    A,Rr           Exchange A and register

XCH    A,@Ri          Exchange A and data memory

XCHD   A,@Ri          Exchange nibble of A and register

MOVX   A,@Ri          Move external data memory to A   *

MOVX   @Ri,A          Move A to external data memory   *

MOVP   A,@A           Move to A from current page

MOVP3  A,@A           Move to A from page 3

Timer/Counter

MOV    A,T            Read timer/counter

MOV    T,A            Load timer/counter

STRT   T              Start timer

STRT   CNT            Start counter

STOP   TCNT           Stop timer/counter

EN     TCNTI          Enable timer/counter interrupt

DIS    TCNTI          Disable timer/counter interrupt

Control

EN     DMA            Enable DMA handshake lines       **

EN     FLAGS          Enable master interrupts         **

EN     I              Enable external interrupt        *

EN     I              Enable IBF interrupt             **

ENT0   CLK            Disable timer/counter interrupt  *

DIS    I              Disable external interrupt       *

DIS    I              Disable IBF interrupt            **

SEL    RB0            Select register bank 0

SEL    RB1            Select register bank 1

SEL    MB0            Select memory bank 0             *

SEL    MB1            Select memory bank 1             *

NOP                   No operation

IDL                   Select idle operation

The  IDL instruction applies only to those devices having   a low

power operation mode.

8. Reference books

CP/M Operating System Manual  Digital Research

INTEL Microcontroller Handbook 1986  (210918)

INTEL Microsystem Components Handbook 1986  (230843)

----------